ATC 800 Series Measurement Notes and Test Conditions

This describes the applicable test conditions and equipment used in the measurement of scattering parameters for the ATC 800 series capacitors.

Each capacitor, with the exception of 800B devices, was horizontally mounted -- electrodes parallel to the plane of the circuit board -- in a series configuration on microstrip. The 800B units were mounted vertically (electrodes perpendicular to the plane of the circuit board). As shown below, each device under test (DUT) subtended a “G”-mil gap in a “W”-mil wide trace on Rogers RO4350 & TMM-3 softboard, “T” mils thick.  “G,” “W,” and “T” are given in Table 1 for various-size capacitors.  In general, “W” was kept at about the width of the capacitor (thickness for 800R), with “T” selected to make the trace width a 50-ohm line.

W

All dimensions in mils

trace (1/2 oz. Cu)

G

T

Rogers Softboard

reference

planes

capacitor under test

| Case Size / EIA Size | G (mils) | W (mils) | T (mils) | Softboard |
| --- | --- | --- | --- | --- |
| “A” - 0605 | 35 | 52 | 23.3 | RO4350 |
| “B” - 1210 | 80 | 52 | 23.3 | RO4350 |
| “R” - 0810 | 40 | 80 | 35.0 | TMM-3 |

All measurements were made using an Anritsu 3680K Universal Test Fixture and an Agilent - N5230A Vector Network Analyzer having a four-receiver architecture.  Measurements have been de-embedded to the edges of the capacitor under test using a standard TRL calibration procedure.  (I.e., S-parameters are referenced at the edges of each capacitor.)

Data is taken at 401 points (801 for < 10.0 Ghz) for each capacitor value.  The following is a data sample:

# GHZ S DB R 50

0.05   -3.79  -50.89   -2.26   39.32   -2.26    39.32   -3.79   -50.89

0.18  -13.22  -78.27   -0.21   12.01   -0.21    12.01   -13.22  -78.27

0.31  -17.93  -84.05   -0.05    6.33   -0.05    6.33    -17.93  -84.05

0.45  -21.27  -87.04   -0.02    3.86   -0.02    3.86    -21.27  -87.04

The first line is a header, designated by the “#” character. Succeeding characters describe respectively the frequency units, parameter, measurement format, and characteristic impedance of the measurement system (in this case 50 ohms).

The data is arranged as follows: The first column is the frequency in GHz.  The next two columns are S11 magnitude (in db) and S11 phase. Succeeding column pairs represent S21, S12, and S22

Definitions –

First Series Resonance (FSR) is the lowest frequency at which the input reactance, referenced at the capacitor edge, becomes zero ohms -- provided that, up to that point, both real and reactive parts of the input impedance exhibit essentially monotonic behavior.  Should the monotonicity condition be violated, the FSR shall be undefined.

First Parallel Resonance (FPR) is the lowest frequency at which there is a notch or suck out in the S21 magnitude response.

DISCLAIMER: American Technical Ceramics (ATC) has made every effort to

make this information as accurate as possible.  However, no responsibility

is assumed by ATC for its use, nor any infringements of rights of third

parties which may result from its use.  ATC reserves the right to revise

the content or modify its product line without prior notice.