ATC 0603 Attenuator Measurement Notes and Test

This describes the applicable test conditions and equipment used in the measurement of scattering parameters for the ATC 0603 Attenuators.

Note that S-parameters are dependent on the particular substrate (dielectric constant and thickness) as well as the pad/trace.  Unless the conditions of use are very similar to those in ATC’s standard measurement setup, or the operating frequency is so low that none of the above matters very much, there is the possibility that S-parameter measurements may be more misleading than helpful.

In ATC’s test setup, each Attenuator was mounted in a series configuration on microstrip.  As shown below, each unit under test (UUT) subtended a “G”-mil gap in a “W”-mil wide trace on Rogers 4350 softboard, “T” mils thick.  “G,” “W,” and ”T” are given in Table 1 for the 0603 attenuator.  In general, “W” was kept at about the width of the attenuator, with “T” selected to make the trace width a 50-ohm line.

Table 1

| Case Size | G (mils) | W (mils) | T (mils) |
| --- | --- | --- | --- |
| 0603 | 44 | 30 | 13.3 |

All measurements were made using an Anritsu 3680K Universal Test Fixture and a Vector Network Analyzer having a four-receiver architecture.  Measurements have been de-embedded to the edges of the attenuator under test using a standard TRL calibration procedure.  (I.e., S-parameters are referenced at the edges of each attenuator.)

The following is a data sample:

| ! s2p File. Ports: 1,2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # MHz S RI R 50 |  |  |  |  |  |  |  |  |
| 500 | -0.00263 | 0.009926 | 0.573218 | -0.03405 | 0.572765 | -0.03296 | 0.003957 | 0.015731 |
| 597.5 | -0.00158 | 0.008358 | 0.572273 | -0.03993 | 0.572355 | -0.03931 | 0.007561 | 0.01996 |
| 695 | 0.001127 | 0.008667 | 0.571491 | -0.04599 | 0.571277 | -0.04606 | 0.005402 | 0.02502 |
| 792.5 | 0.001686 | 0.010647 | 0.570677 | -0.05205 | 0.570453 | -0.05194 | 0.006316 | 0.022803 |
| 890 | -0.00043 | 0.012719 | 0.569315 | -0.05801 | 0.569313 | -0.05763 | 0.010528 | 0.021342 |
| 987.5 | -0.00239 | 0.010135 | 0.56818 | -0.06345 | 0.568145 | -0.06356 | 0.020342 | 0.024808 |
| 1085 | 0.00174 | 0.007724 | 0.566842 | -0.06934 | 0.566897 | -0.06934 | 0.014405 | 0.044144 |

The first line is a comment and denotes the file type and VNA measurement ports.  The second line is a header, designated by the “#” character. Succeeding characters describe respectively the frequency units, parameter, measurement format, and characteristic impedance of the measurement system (in this case 50 ohms).  The data is arranged  as follows: The first column is the frequency in Hz.  The next two columns are S11 magnitude (in real / Imaginary). Succeeding column pairs represent S21, S12, and S22.

The lowest measured frequency is 500 MHz.  An artificial point at 0 frequency can be added to some scattering matrices to avoid problems encountered when using certain software simulators.  The highest measured frequency varies; it is generally selected to include at least the first parallel resonance frequency (see below), and often the second.  In general, customers are advised to be cautious when using software simulation since many programs attempt to extrapolate measured data both below and above actual measurement frequencies – and often arrive at grossly incorrect results.  If in doubt, customers are advised to examine particular scattering matrices in a word processor or other suitable program to determine actual upper and lower measured frequencies.

For passive devices, the magnitude of each S-parameter must be equal to or less than unity, i.e., when expressed in dB, it must be    However, because of small, unavoidable errors in the TRL calibration, the magnitudes of some measured S-parameters near zero-dB are on rare occasions slightly positive.  In the published scattering matrices, each of these elements is assigned a value of  -0.01 dB.

DISCLAIMER: American Technical Ceramics (ATC) has made every effort to

make this information as accurate as possible.  However, no responsibility

is assumed by ATC for its use, nor any infringements of rights of third

parties which may result from its use.  ATC reserves the right to revise

the content or modify its product line without prior notice.