ATC WL Series Scattering Matrix Measurement Notes and Test Conditions

This describes the applicable test conditions and equipment used in the measurement of scattering parameters for the ATC 0402, 0603, 0805, and 1008 WL series inductors.

Each inductor was mounted in a series configuration on microstrip.  As shown below, each unit under test (UUT) subtended a “G”-mil gap in a “W”-mil wide trace on softboard “T” mils thick.  “G,” “W,” “T,” and the softboard dielectric constant are given in Table 1 for various-size inductors.  In general, “W” was kept at about the width of the inductor, with “T” selected to make the trace width a 50-ohm line.

| Case Size | G (mils) | W(mils) | T(mils) | Softboard | Dielectric Constant |
| --- | --- | --- | --- | --- | --- |
| 0402 | 24 | 22 | 10 | Rogers 4350 | 3.48 |
| 0603 | 36 | 29 | 13.3 | Rogers 4350 | 3.48 |
| 0805 | 46 | 52 | 23.3 | Rogers 4350 | 3.48 |
| 1008 | 50 | 80 | 35 | Rogers TMM3 | 3.27 |

Measurements on the 0402, 0603, and 0805 case sizes were made using an HP8722D Vector Network Analyzer; measurements on the 1008 case size were made using an Agilent E8358A Vector Network Analyzer.  Both VNAs had a four-receiver architecture.   All measurements used an Anritsu 3680K Universal Test Fixture and have been de-embedded to the edges of the UUT.  De-embedding for the 0402, 0603, and 0805 case sizes was accomplished by using a standard TRL calibration; de-embedding for the 1008 case size employed a 3.5 mm connector SOLT calibration and subsequent re-referencing in microstrip to the edges of the inductor under test.

Data is taken at 401 points for the 0402, 0603, and 0805 case sizes, and 801 points for the 1008 case size.  The following is a data sample:

#  HZ  S  DB  R  50

400000000  -10.812    71.292   -.4176   -17.821   -.4227   -17.915   -10.767    71.097     422500000  -10.384    70.343   -.4643   -18.784   -.4605   -18.844   -10.334    70.248

445000000  -9.9713    69.478   -.5057   -19.74     -.5039   -19.778   -9.9209    69.355

The first line is a header, designated by the “#” character. Succeeding characters describe respectively the frequency units, parameter, measurement format, and characteristic impedance of the measurement system (in this case 50 ohms).

The data is arranged as follows: The first column is the frequency in Hz.  The next two columns are S11 magnitude (in db) and S11 phase angle (in degrees).  Succeeding column pairs represent S21, S12, and S22.

DISCLAIMER: American Technical Ceramics (ATC) has made every effort to

make this information as accurate as possible.  However, no responsibility

is assumed by ATC for its use, nor any infringements of rights of third

parties which may result from its use.  ATC reserves the right to revise

the content or modify its product line without prior notice.